# Service report — SD50B-2026-10005 — 2026-07-10

Document service-report-sd50b-2026-10005-10 · revision 1 · effective 2026-07-10

Machine types: SD-50B

Item numbers: 1.512-341.0

Serial numbers: SD50B-2026-10005

Site: riverside-mall

## Findings

Found shrink wrap wound tightly around the brush drive shaft on SD50B-2026-10005. Bristle length measured within limits once the film was removed, so the overload was mechanical rather than wear.

## Work carried out

Replaced the panel filter and its seal; the old seal had taken a compression set.

Cleaned the float shut-off and the recovery tank breather, then flushed the suction path.

Replaced the squeegee blade set and reset the deflection across the full width.

## Parts used

Float shut-off assembly

Drive plate

## Outcome

Left out of service pending a replacement pack. The site has been told not to return it to the floor.