# Service report — SD50B-2026-10416 — 2026-01-25

Document service-report-sd50b-2026-10416-09 · revision 1 · effective 2026-01-25

Machine types: SD-50B

Item numbers: 1.512-342.0

Serial numbers: SD50B-2026-10416

Site: hangar-seven

## Findings

Found shrink wrap wound tightly around the brush drive shaft on SD50B-2026-10416. Bristle length measured within limits once the film was removed, so the overload was mechanical rather than wear.

## Work carried out

Cleaned the float shut-off and the recovery tank breather, then flushed the suction path.

Advised the site on the charging temperature limit and left a copy of the procedure.

Replaced the squeegee blade set and reset the deflection across the full width.

## Parts used

Drive plate

Float shut-off assembly

## Outcome

Returned to service with a note on the machine log to re-measure the brushes at the next interval.